QT SERIES OLDER INTERFACE

HOW TO SETUP INTERNET REMOTE ACCESS VIA DMZ

- Go to Main Menu and go to Setup

- Go to Network

- Enable Obtain IP automatically – then click Apply. Make note on IP address

- Login to your Router from a computer.

- Find DMZ setting in the Router.

You can call your Router support or your ISP

Or you can Google: Where is DMZ setting in Netgear Router (example)

- Enable DMZ for the IP address of the DVR ( from step 3 above).

This an example of enabling DMZ in Frontier Router.

- From a computer on the same network as the DVR, go to www.canyouseeme.org

Your IP is your Internet IP address.

Port to check:  85 (HTTP port as listed above) – Check port – Success

Then check one more port: 6036 (Server port)

- If both ports are success, then you can use the IP address listed in canyouseeme to remote access your system from Phone or Computer.

- Example how to set the QTVIEW App on your phone: